NPS DSC Division 1 Specifications

National Park Service (NPS) - Denver Service Center (DSC) | 1-27-21

By using NPS DSC Division 1 Specifications, you acknowledge you read, understand,
and will format edits according to NPS DSC Division 1 Specifications Instructions.

<Insert Park and Project Management Information System (PMIS) number>

Specification is required if rock removal by drilling and blasting is allowed. Section is only a guide and needs editing and revision by Architect/Engineer (A/E) technical specialty. Coordination with Section 01 35 23 "Safety Requirements" is required to fully address safety requirements. Contact David Karplus in Intermountain Region (IMR).

SECTION 01 35 25 - EXPLOSIVES

GENERAL

DESCRIPTION

Section consists of activities relating to explosives, including receiving, handling, transporting, storing, distributing, priming, loading, firing, and disposal.

QUALITY ASSURANCE

Regulatory Agencies: Comply with Occupational Safety and Health Administration (OSHA) regulations contained in Code of Federal Regulations (CFR) 29 CFR 1910 and 1926; Department of Transportation (DOT) rules and regulations contained in 14 CFR 103, 46 CFR 146-149, and 49 CFR 390-397; and Bureau of Alcohol, Tobacco, Firearms, and Explosives (ATF) regulations contained in 27 CFR 55.

Blaster-In-Charge: One competent, experienced person shall be specifically designated in charge of explosives and related activities. Blaster-In-Charge shall carefully supervise work related to the use, storage, transportation, and handling of explosives. Blaster-In-Charge or Contractor shall select a blasting crew to assist with explosives activities, including transportation and area security. Only a minimum number of competent, experienced personnel, consistent with efficient operation, shall be permitted to handle explosives. Anyone demonstrating carelessness, incompetence, or inexperience shall be excluded from further handling of explosives.

SUBMITTALS

Blaster-In-Charge Qualifications:

Proof of a valid state Blaster's License or other license issued by an equivalent licensing body.

Written resume showing three or more years of active involvement as blaster-in-charge on projects similar in scope to this contract.

Five references who can testify to the known qualifications and reliability of proposed Blaster-In-Charge

Blasting Crew Qualifications: Proof that crew members have completed explosives and blasting safety training of at least 24 hours and/or have one or more years of acceptable experience.

Blasting Plan:

Transportation: Include description and license number of vehicle to be used, routes to be traveled, proposed hours of travel, and qualifications of driver.

Storage and Magazines: Describe proposed inventory system; show location and construction of magazine and day-boxes and warning signs.

Blast Site Operations: Include type of explosives, initiation system, drilling system, loading plan, firing plan, pre-blast and post-blast inspection, handling of misfires, and removal and disposal of excess explosives.

Area Security: Include proposed signs, guard system, signal system, methods of communication, and pre-blast notification of affected agencies or entities.

Permits and Licenses: Submit copies or other proof of applicable permits and licenses.

Daily Submittal: Copies of blast records shall be submitted to the Contracting Officer (CO) daily. Include:

Types and amount of powder, drill pattern, loading, weather, initiation system.

Names of blast crew, time of shot(s), results.

Ground vibration monitoring results, if required.

Types and proof of noise monitoring.

PROJECT CONDITIONS

Protection: Safety of personnel shall be controlling consideration in decisions involving use of explosives. Use blasting mats and other protective devices and additional precautions necessary to prevent damage to landscape features and structures. Should damage occur, make restoration as required by the Contracting Officer at no additional expense to the Government.

Fire Danger: During periods of fire danger, Contracting Officer may require a person be detailed as a fire watch for one hour after blasting, or for two hours during periods of extreme fire danger. Under extreme fire danger, use of all-electric detonation systems may be required by Contracting Officer.

CLOSEOUT SUBMITTALS

As specified in Section 01 77 00 "Closeout Procedures."

On completion of work, furnish written statement, countersigned by Blaster-In-Charge, certifying:

Blast holes loaded with explosives and any other set explosive charges have either been detonated, unloaded, or disposed of in an approved manner.

Blasting is complete.

Explosive materials, including detonators, detonating cord, explosives, and unmixed components of a two-component explosive system, have been removed from the Park.

PRODUCTS

GENERAL

Provide as required. Use only explosives, explosive components, and detonators commercially manufactured within the previous two years.

EXPLOSIVES

Products containing nitroglycerin shall not be used without a variance from the National Park Service (NPS) Regional Blasting Officer or the Denver Service Center (DSC) Blasting Officer.

INITIATION SYSTEMS

Cap and fuse initiation is specifically prohibited without a variance from the NPS Regional Blasting Officer or the DSC Blasting Officer.

Electrical Initiation Circuits: Parallel circuit systems are prohibited. Series circuits are authorized if included in approved blasting plan.

EXECUTION

GENERAL

Contractor shall abide by referenced rules and regulations and shall give special attention to following specific rules:

Locate magazines only at sites approved by Contracting Officer.

Magazines shall be of the type required by Bureau of Alcohol, Tobacco, Firearms, and Explosives (ATF) for material being stored.

Keep no more than a one-day supply of explosives and detonators at or near work site.  Keep explosives and detonators in separate approved day-boxes.

Provide warning signs at access points, including warnings to turn off radio transmitters near electric blasting, in accordance with Occupational Safety and Health Administration (OSHA) requirements.

Daily explosives inventory records shall be kept, and a record or log shall be made of each blast. Records shall be available for inspection by Contracting Officer upon request.

Magazines and day-boxes shall be kept locked at all times except when withdrawing or returning materials from or to that magazine or box.

Clear blast area of unnecessary personnel and equipment before delivery of explosives to the site.

Provide a positive signal system to give adequate warning in every direction. Guard access points, including water access, to blast area to halt personnel and vehicles a safe distance from the blast. Maintain intercommunication between guards and person firing the blast, ensuring blast area is clear prior to firing. Guarding shall begin when loading begins and shall continue until post-blast inspection has been made and "all clear" signal given.

Use only wooden or plastic tamping poles for charging explosives into drill holes. Drilling in bootlegs or within 50 feet of any loaded hole is prohibited.

Use only approved blasting machines to initiate shots.

END OF SECTION 01 35 25