NPS DSC Division 1 Specifications

National Park Service (NPS) - Denver Service Center (DSC) | 1-27-21

By using NPS DSC Division 1 Specifications, you acknowledge you read, understand,
and will format edits according to NPS DSC Division 1 Specifications Instructions.

<Insert Park and Project Management Information System (PMIS) number>

SECTION 01 57 19.12 – NOISE AND ACCOUSTICS MANAGEMENT

Section includes requirements for noise management. Refer to ASTM E1433 for an overview of the ASTM standards relative to environmental acoustics.

Edit to suit location and project; include specific equipment and dB(A) (A-weighted decibels) readings as required.

GENERAL

SUMMARY

Section includes:

Special requirements for noise and acoustics management during [deconstruction] [renovation] [construction] operations.

DEFINITIONS

Ambient noise level: The total noise associated with a given environment, being usually a composite of normal or existing sounds from all sources near and far, excluding the noise source at issue.

Daytime: The hours from 7 A.M. to 9 P.M. on weekdays and 9 A.M. to 9 P.M. on weekends and holidays.

Nighttime: All non-daytime hours.

Property line: The real or imaginary line along the ground surface and its vertical extension, which separates real property owned or controlled by one person from contiguous real property owned or controlled by another person or from any public right-of-way or from any public space.

Receiving noise area: Any real property where people live or work and where noise is heard, excluding the project or source area.

PRODUCTS (Not Used)

EXECUTION

NOISE MANGEMENT

Noise Control: Perform [deconstruction] [renovation] [construction] operations to minimize noise. Perform noise-producing work in less sensitive hours of the day or week as directed by the Contracting Officer CO).

Repetitive and/or intermittent, high-level noise: Permitted only during Daytime.

Do not exceed the following dB(A) limitations at 50 feet:

Sound Level in dB(A)	Time Duration of Impact Noise

70	More than 12 minutes in any hour

80	More than 3 minutes in any hour

Maximum permissible construction equipment noise levels at 50 feet:

EARTHMOVING	dB(A)	MATERIALS HANDLING	dB(A)

Front Loaders	75	Concrete Mixers	75

Backhoes	75	Concrete Pumps	75

Dozers	75	Cranes	75

Tractors	75	Derricks Impact	75

Scrapers	80	Pile Drivers	95

Graders	75	Jack Hammers	75

Trucks	75	Rock Drills	80

Pavers, Stationary	80	Pneumatic Tools	80

Pumps	75	Saws	75

Generators	75	Vibrators	75

Compressors	75

Ambient Noise:

Maximum noise levels (dB (decibel)) for receiving noise area at property line shall be as follows:

Residential receiving area							Daytime:	65 dB

Nighttime:	45 dB

Commercial/Industrial receiving area					Daytime:	67 dB

Nighttime:	65 dB

In the event the existing local ambient noise level exceeds the maximum allowable receiving noise level (dB), the receiving noise level maximum for construction operations shall be adjusted as follows:

Residential receiving area: Maximum 3 additional dB above the local ambient as measured at property line.

Commercial/Industrial receiving area: Maximum 5 additional dB above the local ambient as measured at the property line.

FIELD QUALITY CONTROL

Assess potential effects of construction noise on [adjacent neighbors] [facility occupants] in accordance with ASTM E1686 and as follows:

Ambient noise measurement: Measure at property line at a height of at least four (4) feet above the immediate surrounding surface. Average the ambient noise level over a period of at least 15 minutes.

Ambient noise measurement at urban sites: Conduct during morning peak traffic hour between 7 A.M. and 9 A.M. and afternoon peak traffic hour between 4 P.M. and 6 P.M.  In addition, conduct a 24-hour measurement at the proposed project site to document the noise pattern throughout the day. Adjust and weight for seasonal and climatic variations.

Monitor noise produced from construction operations in accordance with ASTM E1780.

END OF SECTION 01 57 19.12