NPS DSC Division 1 Specifications

National Park Service (NPS) - Denver Service Center (DSC) | 9-29-22

By using NPS DSC Division 1 Specifications, you acknowledge you read, understand,
and will format edits according to NPS DSC Division 1 Specifications Instructions.

<Insert Park and Project Management Information System (PMIS) number>

SECTION 01 73 40 - EXECUTION

GENERAL

SUMMARY

Section includes general procedural requirements governing execution of Work including:

Adjust list to suit Project.

Coordination with utility service providers

Construction layout

Field engineering and surveying

General installation of products

Progress cleaning

Starting and adjusting

Protection of installed construction

Correction of the Work

SUBMITTALS

Certificates: Submit certificate signed by [land surveyor] [professional engineer] certifying location and elevation of improvements comply with requirements.

Landfill Receipts: Submit copy of receipts issued by landfill facility, licensed to accept hazardous materials, for hazardous waste disposal.

Certified Surveys: Submit [two] <Insert number> copies signed by [land surveyor] [professional engineer].

Quantity Surveys: Submit [2] <Insert number> copies showing quantities of work performed and actual construction completed in place.

QUALITY ASSURANCE

Land Surveyor Qualifications: Professional [land surveyor] [professional engineer] legally qualified to practice in jurisdiction where Project is located and is experienced in providing land-surveying services of kind indicated.

PRODUCTS (Not Used)

EXECUTION

EXAMINATION

Retain paragraph and subparagraph below for renovation work. Revise to suit Project.

Existing Conditions: Existence and location of site improvements and other construction indicated as existing are not guaranteed.

Before beginning work, investigate and verify the existence and location of mechanical and electrical systems and other construction affecting the Work.

Before construction, verify the location and points of connection of utility services.

Revise first paragraph and subparagraphs below to suit Project. Verify with National Park Service (NPS) and utility companies. Coordinate with Division 33 Sections.

Existing Utilities: Existence and location of underground and other utilities and construction indicated as existing are not guaranteed. Before beginning sitework, investigate and verify existence and location of underground utilities and other construction affecting Work.

Before construction, verify location and invert elevation at points of connection of sanitary sewer, storm sewer, and water-service piping; and underground electrical services.

Furnish location data for work related to Project that must be performed by public utilities serving Project site.

Acceptance of Conditions: Examine substrates, areas, and conditions, with Installer or Applicator present where indicated, for compliance with requirements for installation tolerances and other conditions affecting performance. Record observations.

Verify compatibility with and suitability of substrates, including compatibility with existing finishes or primers.

Examine roughing-in for mechanical and electrical systems to verify actual locations of connections before equipment and fixture installation.

Examine walls, floors, and roofs for suitable conditions where products and systems are to be installed.

Proceed with installation only after unsatisfactory conditions have been corrected. Proceeding with the Work indicates acceptance of surfaces and conditions.

COORDINATION WITH UTILITY SERVICE PROVIDERS

Check to see who owns and maintains the Utility Services. In some cases NPS may own the Utility Service. If not, NPS may have already contracted with local utility to move or modify their utility service. Coordinate with NPS on who owns and maintains the service, what has been accomplished with local utility providers, and clearly define what the Contractor’s responsibilities will be. Coordinate Design with availability and extent of Utility services. Edit paragraph and subparagraphs below accordingly.

Coordination with Utility Service Providers: Contact following Utility Service providers, sufficiently in advance to avoid delaying the work, to coordinate Contractor’s portion of Work, testing requirements, inspections, [etc.] <Insert other general requirements.>

Electrical: Service Contact: Contact <Insert contact information, name, phone number, etc.> to coordinate Electrical service requirements.

Construction Contractor Responsibilities: Contractor is responsible for <Insert specific responsibilities > <Add specification and/or drawing references>.

Water Service Contact: Contact <Insert contact information, name, phone number, etc.> to coordinate Water service requirements.

Construction Contractor Responsibilities: Contractor is responsible for <Insert specific responsibilities > <Add specification and/or drawing references>.

Wastewater Service Contact: Contact <Insert contact information, name, phone number, etc.> to coordinate Wastewater service requirements.

Construction Contractor Responsibilities: You are responsible for <Insert specific responsibilities > <Add specification and/or drawing references>.

Telephone Service Contact: Contact <Insert contact information, name, phone number, etc.> to coordinate Telephone service requirements.

Construction Contractor Responsibilities: You are responsible for <Insert specific responsibilities > <Add specification and/or drawing references>.

PREPARATION

Field Measurements: Take field measurements as required to fit Work properly. Recheck measurements before installing each product. Where portions of Work are indicated fit to other construction, verify dimensions of other construction by field measurements before fabrication. Coordinate fabrication schedule with construction progress to avoid delaying Work.

Space Requirements: Verify space requirements and dimensions of items shown diagrammatically on Drawings.

Review of Contract Documents and Field Conditions: Immediately on discovery of need for clarification of the Contract Documents caused by differing field conditions outside control of Contractor, submit request for information to Contracting Officer in accordance with Section 01 31 00 "Project Management and Coordination."

CONSTRUCTION LAYOUT

Verification: Verify layout information shown on Drawings, in relation to the existing benchmarks before proceeding to lay out Work. Notify Contracting Officer promptly if discrepancies are discovered.

General: Engage a [land surveyor] [professional engineer] to lay out Work using accepted surveying practices.

Establish benchmarks and control points to set lines <and levels at each story> of construction and elsewhere as needed to locate each element of Project.

Establish dimensions within tolerances indicated.  Do not scale Drawings to obtain required dimensions.

Inform installers of lines and levels to which they must comply.

Check location, level and plumb, of every major element as Work progresses.

Notify Contracting Officer when deviations from required lines and levels exceed allowable tolerances.

<Close site surveys with an error of closure equal to or less than the established standard.>

Site Improvements: Locate and lay out site improvements, including pavements, grading, fill and topsoil placement, utility slopes, and invert elevations.

Building Lines and Levels: Locate and lay out control lines and levels for structures, building foundations, column grids, and floor levels, including those required for mechanical and electrical work. Transfer survey markings and elevations for use with control lines and levels. Level foundations and piers from two or more locations.

Record Log: Maintain log of layout control work. Record deviations from required lines and levels. Include beginning and ending dates and times of surveys, weather conditions, name and duty of each survey party member, and types of instruments and tapes used. Make log available for review by National Park Service (NPS).

FIELD ENGINEERING

Reference Points: Locate existing permanent benchmarks, control points, and similar reference points before beginning Work. Preserve and protect permanent benchmarks and control points during construction operations. Controls destroyed by Contractor will be replaced by Contractor at their expense.

Existing Monuments: All benchmarks, land corners, and triangulation points, established by other surveys, existing within construction area shall be preserved. If existing monuments interfere with Work, secure written permission before removing them.

Benchmarks: Establish and maintain a minimum of [two] <Insert number> permanent benchmarks on Project site, referenced to data established by survey control points. Comply with NPS requirements for type and size of benchmark.

Record benchmark locations, with horizontal and vertical data, on Project Record Documents.

Retain first paragraph below if required by NPS.

Certified Survey: On completion of foundation walls, major site improvements, and other work requiring field-engineering services, prepare a certified survey showing dimensions, locations, angles, and elevations of construction and sitework.

INSTALLATION

General:  Locate Work and components of Work accurately in correct alignment and elevation, as indicated.

Make vertical work plumb and make horizontal work level.

Where space is limited, install components to maximize space available for maintenance and ease of removal for replacement.

Conceal pipes, ducts, and wiring in finished areas, unless otherwise indicated.

Comply with manufacturer's written instructions and recommendations for installing products in applications indicated.

Install products at the time and under conditions for best possible results. Maintain conditions required for product performance until Substantial Completion.

Conduct construction operations so no part of the Work is subjected to damaging operations or loading in excess of that expected during normal conditions of occupancy.

Tools and Equipment: Do not use tools or equipment that produce harmful noise levels.

Templates: Obtain and distribute to parties involved templates for work specified to be factory prepared and field installed. Check Shop Drawings of other work to confirm adequate provisions are made for locating and installing products to comply with indicated requirements.

Anchors and Fasteners: Provide anchors and fasteners as required to anchor each component securely in place, accurately located and aligned with other portions of Work.

Mounting Heights: Where mounting heights are not indicated, mount components at heights directed by Contracting Officer.

Allow for building movement, thermal expansion, and contraction.

Coordinate installation of anchorages. Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors to be embedded in concrete or masonry. Deliver to Project site in time for installation.

Joints: Make joints of uniform width. Where joint locations in exposed work are not indicated, arrange joints for best visual effect. Fit exposed connections together to form hairline joints.

Hazardous Materials: Use products, cleaners, and installation materials not considered hazardous.

Quantity surveys: Shall be conducted, and data derived from these surveys shall be used in computing quantities of work performed and actual construction completed and in place.

Contractor shall conduct original and final surveys and surveys for any periods for which progress payments are requested. These surveys shall be conducted under direction of a representative of the Contracting Officer, unless Contracting Officer waives requirement in a specific instance. Government shall make such computations as are necessary to determine quantities of work performed or finally in place. Contractor shall make computations based on surveys for any periods for which progress payments are requested.

Promptly upon completing a survey, Contractor shall furnish originals of field notes and other records relating to survey or layout of Work to Contracting Officer. Contractor shall retain copies of all such material furnished to Contracting Officer.

PROGRESS CLEANING

Article refers to regular cleaning operations conducted while construction is in progress. Requirements for final cleaning before Substantial Completion are included in Section 01 77 00 "Closeout Procedures."

General: Clean Project site, work areas, and common areas daily. Coordinate progress cleaning for joint-use areas where more than one Installer has worked. Enforce requirements strictly. Dispose of materials lawfully.

Comply with requirements in National Fire Protection Association (NFPA) 241 for removal of combustible waste materials and debris at the end of every work session.

Do not hold materials more than 7 days during normal weather or 3 days if the temperature is expected to rise above 80 degrees Fahrenheit (27 degrees Celsius).

Containerize hazardous and unsanitary waste materials separately from other waste. Mark containers appropriately and dispose of legally, according to regulations.

Site: Maintain Project site free of waste materials and debris.

Work Areas: Clean areas where work is in progress to level of cleanliness necessary for proper execution of Work.

Remove liquid spills promptly.

Where dust would impair proper execution of Work, broom-clean or vacuum entire work area, as appropriate.

Contractor shall provide progress cleaning that minimizes sources of food, water, and harborage available to pests.

Installed Work: Keep installed work clean. Clean installed surfaces according to written instructions of manufacturer or fabricator of product installed, using only cleaning materials specifically recommended. If specific cleaning materials are not recommended, use cleaning materials not hazardous to health or property and will not damage exposed surfaces.

Executive Order (EO) 13423 includes requirements for Federal Agencies to reduce "the quality of toxic and hazardous chemicals and materials acquired, used, or disposed of by the agency".

Utilize non-toxic cleaning materials and methods.

Comply with Green Seal Standard (GS) 37 for general purpose cleaning and bathroom cleaning.

Use natural cleaning materials where feasible. Natural cleaning materials include:

Abrasive cleaners:  substitute 1/2 lemon dipped in borax.

Ammonia: substitute vinegar, salt and water mixture, or baking soda and water.

Disinfectants: substitute 1/2 cup borax in gallon water.

Drain cleaners: substitute 1/4 cup baking soda and 1/4 cup vinegar in boiling water.

Upholstery cleaners: substitute dry cornstarch.

Concealed Spaces: Remove debris from concealed spaces before enclosing the space.

Exposed Surfaces in Finished Areas: Clean exposed surfaces and protect from damage and deterioration at time of Substantial Completion.

Waste Disposal: Do not bury or burn waste materials on-site. Do not wash waste materials down sewers or into waterways.

Clean and protect construction in progress and adjoining materials already in place during handling and installation. Apply protective covering where required to ensure protection from damage or deterioration at Substantial Completion.

Clean and provide maintenance on completed construction as frequently as necessary through remainder of the construction period. Adjust and lubricate operable components to ensure operability without damaging effects.

Limiting Exposures: Supervise construction operations so that no part of construction completed or in progress, is subject to harmful, dangerous, damaging, or deleterious exposure during construction period.

Final Cleaning: At completion of Work, remove remaining waste materials, rubbish, tools, equipment, machinery and surplus materials. Clean exposed surfaces and leave Project clean and ready for occupancy.

Provide final cleaning in accordance with ASTM E1971.

STARTING AND ADJUSTING

Start equipment and operating components to confirm proper operation. Remove malfunctioning units, replace with new units, and retest.

Adjust operating components for proper operation without binding. Adjust equipment for proper operation.

Test each piece of equipment to verify proper operation. Test and adjust controls and safeties.  Replace damaged and malfunctioning controls and equipment.

Manufacturer's Field Service: If factory-authorized service representative is required to inspect field-assembled components and equipment installation, comply with qualification requirements in Section 01 40 00 "Quality Requirements."

PROTECTION OF INSTALLED CONSTRUCTION

Provide final protection and maintain conditions that ensure installed Work is without damage or deterioration at time of Substantial Completion.

Comply with manufacturer's written instructions for temperature and relative humidity.

CORRECTION OF THE WORK

Repair or remove and replace defective construction. Restore damaged substrates and finishes. Comply with requirements in Section 01 73 29 "Cutting and Patching."

Repairing includes replacing defective parts, refinishing damaged surfaces, touching up with matching materials, and properly adjusting operating equipment.

Restore permanent facilities used during construction to specified condition.

Remove and replace damaged surfaces exposed to view if surfaces cannot be repaired without visible evidence of repair.

Repair components that do not operate properly. Remove and replace operating components that cannot be repaired.

Remove and replace chipped, scratched, and broken glass or reflective surfaces.

END OF SECTION 01 73 40